Re-Code for Dazzle Headlights (LED Adaptive Headlights like EU-Spec):

Change FA (VO) to Remove  5AP & 8S4 from VO

VO Code FRM Module

Read FA(VO)Activate FA(VO)Read SVTRight-Click ECU & Select Code

VO Code KAFAS2 Module

Right-Click ECU & Select Code

Now Proceed to code FRM with the 2 lines of code needed to complete Euro LED Adaptive Lighting

Re-Code for Enhanced Bluetooth:

Change FA (VO) to Remove 6NH from VO & Add 6NS to VO

VO Code HU_NBT

Read FA(VO)Activate FA(VO)Read SVTRight-Click ECU & Select Code

Re-Code for Sports Automatic Transmission:

Change FA (VO) to Remove 205 from VO & Add 2TB to VO

VO Code EGS,  ICM, & KOMBI

Read FA(VO)Activate FA(VO)Read SVTRight-Click ECU & Select Code

IF YOU NEED TO DO EITHER OF THE ABOVE … FOLLOW THE INSTRUCTIONS FOR “HOW TO CHANGE FA” in my BMW Coding Beginner’s Guide.  This needs to be done FIRST … meaning before you perform FDL Coding or Changing Werte Values

ACSM Module:

SeatBelt Reminder Lights off in Instrument Panel:

3000Gurtzustandsanzeige_Beifahrer_GFW_GZA_BF	set to: nicht_aktiv

3000Gurtzustandsanzeige_Fahrer_GFW_GZA_FA	set to: nicht_aktiv

Disable Welcome 3 Gongs:

3000  Initialwarnung_GWF_IW				set to: nicht_aktiv

SeatBelt Reminders/Sound OFF (Lines below depend on Build Date):

3000SBR_BF_GWF_SBR_BF				set to: nicht_aktiv

3000SBR_FA_GWF_SBR_FA				set to: nicht_aktiv

3000SPW_BF						set to: nicht_aktiv

3000SPW_FA						set to: nicht_aktiv

--OR--

3000SBR_PreWarning_Fahrer				set to: nicht_aktiv

3000SBR_PreWarning_Beifahrer				set to: nicht_aktiv

3000SeatBeltReminder_SBR_Fahrer			set to: nicht_aktiv

3000 SeatBeltReminder_SBR_Beifahrer			set to: nicht_aktiv

ASD Module:

Disable Active Sound Design (///M5 Only)

3000 Model Range 					set to: F021

CAS Module:

Auto Start/Stop OFF by default

3000TC_MSA_DEFAULT_OFF				set to: aktiv – werte=01

Auto Start/Stop – Remember LAST Setting

3000TC_MSA_MEMORY					set to: aktiv – werte=01

Unlock all doors with engine off (Single-Pull Handles):

3002ER-KEYOUT_AUTOVR				set to: aktiv – werte=01

Electronics off when door opened

3000TC_LOGIC_KLR_OFF_DOOR				set to: aktiv

Close Mirrors/Windows/SunRoof from FOB or CA

*** ALSO REQUIRES 1 VALUE CHANGED IN FRM MODULE FOR THIS TO BE ACTIVATED ***

3003KOMFORTSCHLIESSUNG_FB			set to aktiv – werte=00

3003KOMFORTSCHLIESSUNG_PA			set to aktiv – werte=00

Set Mirror Fold Delay Time

3003KMFRT_SCHLIESSEN				set to werte=00

Start Car by pushing button (hold brake not needed)

New Procedures is as mentioned:
- press once for iDrive and fuel pump
- press a second time and hold to crank engine

3000TC_STARTLOCK_BRAKE				set to: nicht_aktiv

3000TC_STARTLOCK_DRIVINGREADINESS		set to: nicht_aktiv

EGS Module:

Enable Launch Control

3000LC							set to: aktiv

FRM Module:

Additional Coding Needed for Euro Adaptive LED Headlight Coding

3080  LAMP_MAP_PARA_SATZ_01			set to: BV_F010_PL_L_ueber_POL_L

3080  LAMP_MAP_PARA_SATZ_02			set to: BV_F010_PL_R_ueber_POL_R

Angel-Eyes Full Brightness (with headights on)

3060U_EFF_POL						set to: wert_05__13,0V

Close Mirrors/Windows/SunRoof from FOB or CA

3020ASP_BEIKLAPPEN_BEI_KOMFORTSCHLIESSEN	set to aktiv

Tilt Passenger Mirror farther down when in reverse

3020ASP_BORDSTEINAUTOMATIK_DELTA		Change Werte to: 4B

Opening FRONT doors won’t interrupt windows:

3030FH_TUER_AUF_STOP_MAUT      	            		set to: nickt_aktiv

Rear Tails & Angel Eyes on as DRL

3050DRL_MODUS						set to: drl_s

Fog Lights stay on w/ High Beams

3050NSW_AUS_BEI_FL			          		set to: nicht_aktiv

Ambiance Lighting controlled independent from Dimmer Switch

####  AMBIENTE_NACHFUEHRUNG			set to: nicht_aktiv

Ambiance Lighting Brightness Settings:

CS_AMB_BRIGHTNESS
Kommentar=Ambient brightness Obergrenze für die Umgebungshelligkeit
Comment = Ambient brightness Upper limit for the ambient brightness 
Default Value = 64 (seeing as the below setting is a HEX value, thus I assume this setting is also in HEX, which makes this 100)

CS_AMB_MIN_BRIGHTNESS
Kommentar=Min Amb brightness Untergrenze für die Umgebungshelligkeit
Comment = Min Amb brightness Lower limit of the ambient brightness
Default Value = 0A (In decimal this is 10)

CS_AMB_MAX_OFFSET
Kommentar=Max amb brightness offset Helligkeit für PWM-Ausgang in Prozent physikalischen Wert 20% ... 80%
Comment = Max amb brightness offset Brightness for PWM output as a percentage physical value of 20% ... 80% 
Default Value = 30

Brake-Force Flashing on Hard Braking:

3050ESS_ERSCHEINUNGSBILD				set to: bremslicht blinkend – werte=02

3050ESS_ON_VERZ					set to: US – werte=0A

3050ESS_GESCHW_SCHWELLE				set to: US – werte=05

High-Beam Assistant – Auto-On:

3050FLA_AUTO_AKTIV					set to: automatisch

3050FLA_VERBAUT					set to: aktiv

Welcome Lights – Add Lights to Welcome Lights

Low Beam:	3050WL_FUNKTION_AL				set to: nicht_aktiv – werte=00

High Beam:	3050WL_FUNKTION_FL				set to: nicht_aktiv – werte=00

License Plate: 3050WL_FUNKTION_KZL				set to: hard_on – werte = 02

Fog Lights:	3050WL_FUNKTION_NSW			set to: hard_on – werte = 02

Tail Lights:	3050WL_FUNKTION_SSL				set to: hard_on – werte = 02

Code for LED Turn-Signal Bulb replace halogens (on Xenon Light Package for LCIs):

FRA_V_L_KALTUEBERWACHUNG 					set to: nicht_aktiv
FRA_V_L_WARMUEBERWACHUNG					set to: nicht_aktiv
FRA_V_L_IS_LED							set to: aktiv
FRA_V_R_KALTUEBERWACHUNG 					set to: nicht_aktiv
FRA_V_R_WARMUEBERWACHUNG					set to: nicht_aktiv
FRA_V_R_IS_LED 							set to: aktiv

HKFM_LS Module:

*** takes up to 30 minutes to begin working after coding ***

Close/Open Trunk from Fob & Kick Panel

3010SCH_FBD						set to: aktiv – werte=01

3010SCH_TOEHKI						set to: aktiv – werte=01

3010TASTER_FBD						set to: aktiv – werte=01

HU_NBT or HU_CIC Module: (Not all available in CIC Headunit)

High-Beam Assistant – Auto-On

3000HIGH_BEAM_ASSISTANT				set to: aktiv

Set cell phone ringer as primary

3001INBAND_RINGING					set to: aktiv

Enable Siri Voice Control

3003CE_DEVICE_SPEECH_RECOGNITION		set to: aktiv

Disable Auto-Locking of Doors once begin driving

3000DOOR_LOCK_AFTER_DRIVING_AWAY		set to: nicht_aktiv

Enable ///M LapTmer

3000  M_LAPTIMER					set to: aktiv

Disable Active Sound Design (///M5 Only)

3000  ASD_SOUND_OFF					set to: aktiv

Enable Active Sound Design Configuration Menu (///M5 Only)

3000  ASD_CONFIGURATION				set to: aktiv

3000  ASD_SOUND_4					set to: aktiv

Enable Sports Displays

3000EFF_DYN_SPORT_CID				set to: aktiv

3000EFF_DYN_SPORT_UNIT				set to: aktiv

* To change units displayed to be ft lb and hp

KOMBI3008DREHMOMENT			set to: lb_ft (displays ft lb)

KOMBI3008LEISTUNG				set to: ps (displays hp)

Enable Volume On-Screen Display:

3002VOLUME_POPUP_DISPLAY				set to: aktiv

Enable Turn Signals in HUD

3001HUD_TURNSIGNAL					set to: aktiv

DVD-in-Motion activated:

3000  SPEEDLOCK_X_KMH_MAX  			Change werte to: FF

3000  SPEEDLOCK_X_KMH_MIN 	 			Change werte to: FF

3000  VIDEO_HANDBRAKE				set to: nicht_aktiv

3000  VIDEO_FRONT_LOCKED				set to: nicht_aktiv

3000  VIDEO_SPEEDLOCK_CONDITION			set to: none=00

Change Navi Voice to British English

HU_CIC3000VOICE_LANGUAGE_RR_US		set to: aktiv

HU_NBT3000LANGUAGE_ENGLISH_UK		set to: master

HU_NBT3000LANGUAGE_ENGLISH_US		set to: nicht_aktiv

Navi Trip-Import

3000NAVI_TRIP_IMPORT					set to: aktiv

Navi Fuel-Stop Proposal Along Route

3000NAVI_FUELSTOP_PROPOSAL			set to: aktiv

Enable Video Files from USB

3000ENT_CODEC_XVID					set to: aktiv

3000ENT_CODEC_XVCD					set to: aktiv

3003API_USB_VIDEO					set to: both

Add GPS Current Location to Navi Menu Items

3000NAVI_POSITION_MENU				set to: aktiv

Add Turn-Signals to HUD

3000HUD_TURNSIGNAL					set to: aktiv

Add Weather Band Radio (CIC Only)

3002RADIO_WEATHERBAND				set to: aktiv

Enable the function of saving video files to NBT Hard Drive for play later

3000  ENT_MC_VIDEO_SUPPORT				set to: aktiv

Disable Office Function Speed Lock-Out

3000 OFFICE_MESSAGES_SPEEDLOCK_CONDITION	set to: nicht_aktiv

Enable Full Text Lines in Office While Driving

3003  PIM_DRIVING_TEXT_LENGTH			set to: whole text_07

Disable Audible Chirp/Beep When Locking Car

3000  ACOUSTICAL_LOCK_CONFIRM			set to: nicht_aktiv

Enable Speech Input for E-Mail or SMS

3003  SPEECH_2_TEXT					set to: aktiv

Enable Time-Stamp Display for Call Log

3003  TIMESTAMP_DISPLAY				set to: aktiv

Add Temperature & PSI to tire monitor screen

3001RDC_SAFETY						set to: aktiv

Disable Legal Disclaimers

3001LEGAL_DISCLAIMER_TIME				set to: kein_ld

3001MACRO_CAM_LEGALDISCLAIMER			set to: kein_ld

3001MACRO_NVICAM_LDISCLAIMER			set to: kein_ld

Change i-Drive Animation to ///M Logo

3001STARTUP_EMBLEM					set to: variant_01

Enable IPCE Web Server @ 172.16.222.1

3000IPCE_WEBSERVER					set to: aktiv

Enable WiFi Hot-Spot

(Need to install a DLNA/UPnP media sharing server on your laptop or phone to be able to use this)

3000ENT_MC_WLAN_STREAM_DMP_COVERART	set to: aktiv

3000ENT_MC_WLAN_STREAM_DMP_VIDEO		set to: aktiv

3000ENT_MC_WLAN_STREAM_DMP			set to: aktiv

3003WLAN_STATUS					set to: aktiv

3003WLAN_MODUS					set to: ap

3003WLAN_SIGNAL_STRENGTH				set to: outdoor

Enable Developer Menu

3000  ENTWICKLER_MENUE				set to: aktiv 01

ICM Module: (Requires Further Testing)

Enable Blind-Spot Detection at LOWER Speeds

3000Hc2_i_CPar_Aktiv_v_low				set to: 25 kmh

--OR--

3000HcSwcCPar_i_Hc2_Aktiv_v_low			set to: 25 kmh

IHKA Module:

A/C System Remembers Last Setting

3000OFF_MEMORY					set to aktiv

Always Remember Air ReCirculating

3000MEMORY_UMLUFT					set to: aktiv

JBBF Module:

Opening REAR doors won’t interrupt windows:

3070FH_TUER_AUF_STOP_MAUT    			set to nickt_aktiv

Disable Headlight Washers

3000  SCHEINWERFERREINIGUNG			set to: nicht_aktiv

Auto Wiper Sensitivity Increase

3530RLS_FLC_SCHWELLWERT_SATZ_1			Change Werte to: 05

3530RLS_FLC_SCHWELLWERT_SATZ_2			Change Werte to: 09

3530RLS_FLC_SCHWELLWERT_SATZ_3			Change Werte to: 0D

Auto Light Sensitivity Increase

3530RLS_DEF_FLC_SCHWELLWERT_SATZ  		set to: option chosen below
3530RLS_FLC_SCHWELLWERT_SATZ 			set to: option chosenbelow

Options:
normal = normal
empfindlich = sensitive
sehr_empfindlich = very sensitive
unempfindlich = insensitive

KAFAS2 Module:

High-Beam Assistant – Auto-On

3050FLA_ON_OFF						set to: aktiv

KOMBI Module:

Show Digital Speed in MPH in Cluster

3000BC_DIGITAL_V					set to: aktiv

Show Turn Signals in HUD

3008HUD_PIA_BLINKER					set to: aktiv

3000BLINKER_HUD_ENABLE				set to: aktiv

Enable GPS-Sync for Clock

3000GPS_UHR						set to: aktiv

Enable HUD for Entertainment & Phone List

*** Adds checkbox under settings for HUD – Displayed Information Menu ***

3000HUD_ENTERTAINMENT_ENABLE			set to: aktiv

3000HUD_TELEFONANRUF_ENABLE			set to: aktiv

3000HUD_TELEFONBUCH_ENABLE			set to: aktiv

3000HUD_ENTERTAINMENTLIST			set to: aktiv

PDC Module:

Enable Rear-View Camera at all Speeds

3009D_SCHWELLE_2					Change werte to: FF

3009V_SCHWELLE_2					Change werte to: FF

SZL Module:

Enable Paddle Shifters (for Retrofits)

3000  Lenkrad_Schaltpaddles				set to: aktiv

TRSVC Module:

Enable Side-View Cameras at all Speeds

3000SV_Activate_Speed_Limit				Change werte to: FF

3000SV_Deactivate_Speed				Change werte to: FF